SECTION 09 84 33

SOUND-ABSORBING WALL UNITS

PART 1 GENERAL

  - SECTION INCLUDES

- Sound-absorbing wall panels, custom-fabricated.

- Related Sections:

- 09 84 36 Sound-Absorbing Ceiling Units

  - REFERENCES

- ASTM International:

- ASTM C423 Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method.

- ASTM E84 Standard Test Method for Surface Burning Characteristics of Building Materials.

- ASTM E795 Standard Practices for Mounting Test Specimens During Sound Absorption Tests.

  - SYSTEM DESCRIPTION

- Performance Requirements:

- Surface Burning Characteristics (ASTM E84):

- Flamespread: 25 maximum.

- Smoke Developed: 450 maximum.

1.04	SUBMITTALS

- General: Submit listed submittals in accordance with Conditions of the Contract and Division 1 Submittal Procedures Section.

- Product Data: Submit product data sheet, for specified products.

- Samples: Submit selection and verification samples of finishes and colors.

- Test Reports: Certified test reports showing compliance with specified performance requirements.

- Standard Systems: Submit copies of previous test reports substantiating performance of system in lieu of retesting.

1.05	QUALITY ASSURANCE

NOTE TO SPECIFIER – Paragraph below should list obligations for compliance with specific code requirements particular to this section. General statements to comply with a particular code are typically addressed in Conditions of the Contract and Division 1 Regulatory Requirements Section. Repetitive statements should be avoided.

- Regulatory Requirements and Approvals: [Specify applicable requirements of regulatory agencies.].

- [Code agency name].

- [Report or approval number].

1.06	DELIVERY, STORAGE, AND HANDLING

- General: Comply with Division 1 Product Requirements Section.

- Delivery: Deliver materials in manufacturer’s original, unopened, undamaged containers with identification labels intact.

- Storage and Protection: Store materials protected from exposure to harmful environmental conditions and at temperature and humidity conditions recommended by the manufacturer.

1.07	PROJECT CONDITIONS

- Environmental Requirements: Do not install panels until wet work, such as concrete and plastering, is complete; the building is enclosed; and the temperature and relative humidity are stabilized at 60 - 80 degrees F (16 - 27 degrees C) and 35% MINIMUM RH and 55% MAXIMUM RH, respectively.

PART 2 PRODUCTS

2.01	SOUND-ABSORBING WALL PANELS

- Manufacturer: Sound Seal Acoustical Products
Poly-Coustix Brand
50 HP Almgren Dr.
Agawam, MA.  USA

Ph. (413) 789-1770
Fax (413) 789-4444
www.soundseal.com

- Substitutions: No substitutions permitted.

2.02	MANUFACTURED UNITS

- Tri-Hex Panels:

- Size: 13” x 15”

- Color: [As selected from panel manufacturer’s stocked range of colors]

- Thickness: 5/8”

- Sound Absorption (ASTM C423): Noise Reduction Co-efficient (NRC) as follows:

  - 9mm panel: .45, minimum.

PART 3 EXECUTION

3.01	MANUFACTURER’S INSTRUCTIONS

- Compliance: Comply with manufacturer’s product data, including product technical bulletins, product catalog installation instructions and product carton instructions for installation.

3.02	EXAMINATION

- Site Verification of Conditions: Verify that substrate conditions, which have been previously installed under other sections, are acceptable for product installation in accordance with manufacturer’s instructions.

- Do not install panels until unsatisfactory conditions are corrected.

3.03	CLEANING

- Follow manufacturer’s instructions for cleaning panels soiled during installation. Replace panels that cannot be cleaned to as new condition.

- Keep site free from accumulation of waste and debris.

END OF SECTION