SECTION 09600.11

SOUND CONTROL UNDERLAYMENTS

[image: image1.jpg]INC2AC TR

IMPACT INSULATION SYSTEMS





PART 1   GENERAL

1.1 SECTION INCLUDES

A. Sound-reducing underlayment for ceramic tile and stone flooring.

1.2 RELATED SECTIONS

.

A. 09 30 13 - Ceramic Tiling & 09 30 16 - Quarry Tiling
1.3 REFERENCES

A. U.S. Department of Housing and Urban Development (HUD): Guide to Airborne, Impact, and Structure Borne Noise Control in Multifamily Dwellings. 

1.4 SUBMITTALS

A. Submit under provisions of Section 01300.

B. Product Data:  Manufacturer's data sheets on each product to be used, including:

1. Preparation instructions and recommendations.

2. Storage and handling requirements and recommendations.

3. Installation methods.

C. Selection Samples:  For each finish product specified, two complete sets of color chips representing manufacturer's full range of available colors and patterns.

1.5 QUALITY ASSURANCE

A. Manufacturer Qualifications: The manufacturer shall be a firm with a minimum of two years of successful experience in manufacture of products with similar requirements.

1. Installer Qualifications: The installer shall be a firm with a minimum of five years of successful experience in installation of products with similar requirements..

1.6 DELIVERY, STORAGE, AND HANDLING

A. Store products in manufacturer's unopened packaging until ready for installation.

B. Store and dispose of solvent-based materials, and materials used with solvent-based materials, in accordance with requirements of local authorities having jurisdiction.

1.7 PROJECT CONDITIONS

A. Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results.  Do not install products under environmental conditions outside manufacturer's absolute limits.

PART 2   PRODUCTS

2.1 MANUFACTURERS

A. Acceptable Manufacturer:  Sound Seal; 50 H.P. Almgren Dr., Agawam, MA 01001.  Toll Free Tel: (800) 569-1294.  Tel: (413) 789-1770.  Fax: (413) 789-2248.  Email: impacta@soundseal.com. Web: http:// www.soundseal.com.

B. Substitutions:  Not permitted.

C. Requests for substitutions will be considered in accordance with provisions of Section 01600.

2.2 SOUND-REDUCING UNDERLAYMENT FOR CERAMIC TILE AND STONE FLOORING

A. Acceptable Product: Impacta CERAZORB as manufactured by Sound Seal.

1. Acoustical Performance Rating (IIC):

a. Wood Structure (IIC) 52.

6” Concrete slab (without ceiling)  (ΔIIC) 20. 

2. Physical Characteristics:

a.
Synthetic cork made from composite bead foam

b. .1875 inch (5 mm) thick.

c. 4 feet (1219 mm) by 4 feet (1219 mm) sheets.

B. Accessories:

1.
T-700 Bond Coat Adhesive.

1. Perimeter isolation material.

(Optional) Acoustical caulk.

PART 3   EXECUTION

3.1 EXAMINATION

A. Do not begin installation until substrates have been properly prepared.

B. If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

3.2 PREPARATION

A. Clean surfaces thoroughly prior to installation.

B. Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

3.3 INSTALLATION

A. Install in accordance with manufacturer's instructions.

3.4 PROTECTION

A. Protect installed products until completion of project.

B. Touch-up, repair or replace damaged products before Substantial Completion.


END OF SECTION


09600.11


